EEE-3207

Digital Signal Processing

Assignment task-1

Obtaining CO2

Analyzing Audio Signals of Distinctive Words Using MATLAB

## Objective:

This assignment aims to give you an experience with hands-on audio signal processing, Fourier analysis, and cross-correlation techniques to differentiate between distinct words based on their frequency characteristics.

You can use any internet sources, tools, resources etc to complete the project. You will be eligible for bonus marking if you can detect all 9 Bangla digits (check the rubrics at the bottom).

# Task Overview:

Record a short audio clip containing two distinct words: "chaar" and "tiin". Analyze the audio signals in MATLAB to observe their frequency characteristics and differentiate between the words based on the following methods:

- Fourier Analysis to extract distinctive frequency components (Fourier coefficient bins).

- Cross-Correlation to detect the characteristic vowel sounds "aa" and " within the words.

# Steps to follow:

- Audio Recording

  - Record three audio clips (in MP3 format) where you and your family members distinctly pronounce the two words "chaar" and tiin". Ensure the recording is clear and free from background noise.

  - Save the audio file with an appropriate name in the same folder as your .m file.

- Preprocessing the Audio

  - Import the audio file into MATLAB using audioread() or any function you want!

  - Extract the two words separately from the audio signal (you may use MATLAB's audio visualization tools to identify time intervals for each word, by plotting or toolboxes, this is NOT necessary for the grading).

  - You can also normalize the signal for consistency if you want.

- Fourier Analysis

  - Apply the Fourier Transform to the signal of each word using the Fast Fourier Transform (FFT). You can also use the manual method instead of FFT. Any method will work.

  - Plot the frequency spectrum of both words.

  - Identify the distinctive frequencies corresponding to the vowel sounds "aa" in "chaar" and

"ii" in "tiin".

  - Discuss in your brief report how the observed frequencies differ and how they can be used to distinguish between the two words.

- Cross-Correlation Analysis

  - Record two more audios with “aa” and “ii”

  - Perform cross-correlation using MATLAB's xcorr() function to compare:

    - The "aa" segment against the words.

    - The "ii" segment against the words.

- Identify peaks in the correlation values and use them to determine which word is being analyzed.

- Explain your results, highlighting the higher correlation values observed.

- Reporting Results

Document following:

- Fourier spectra of each word.

- Key frequencies associated with the vowels "aa" and "ii" (Such as which hertz).

- Cross-correlation plots showing the detected correlations.

## Submission Zip file will contain:

- MATLAB Code: Submit one or multiple script (.m file) containing your entire analysis.

- Read Me file: You know how to run your script, we do not. Write a small read me script to tell me which .m file should I run first and how will I load the audio files and so on.

- Audio Files: All the audios you and your family/friends recorded!

- Plots: Include all relevant plots (Fourier spectra, cross-correlation results) with proper labels and legends.

- Report: Write a concise report (3-6 pages is enough, you can go over but not encouraged) detailing your process, observations, and conclusions. Include figure of key MATLAB outputs.

## Grading Rubrics:

You can use only one method (either Fourier or Cross-Correlation) to do this assignment.

| Criteria | Weight |
| --- | --- |
| Audio Recording | 10% |
| Fourier Analysis | 30% |
| Cross-Correlation Analysis | 30% |
| Coding Quality | 10% |
| Report | 20% |